# Experiment 6: Music Synthesizer

Vineet Deepak Gala

Roll Number 190070072

EE-214, WEL, IIT Bombay

March 31, 2021

## Overview of the experiment:

Aim – To design a music synthesizer that plays a particular set of music notes to make a tune. We have used an FSM for this. First, we have total 33 counts of notes in one cycle of the tune. Depending on current count, we have to assign next note, increment count and loopback from 32 to 0. Diff states in FSM are diff notes we have to play. We also made a clock divider to generate 4Hz clock to track 0.25 second delays between note changed. Transitions occur on +ve edge of this clock.

## Approach to the experiment:

The table with count values:

## Design document and VHDL code if relevant:

toneGenerator: This produces the corresponding tone according to the given input. song_tb: This acts as the testbench code for the whole design music: This is the main design which implements a FSM and finally outputs the needed music and also the LED signal which glows up the correct LED corresponding to the correct note. ntity music is port (toneOut : out std_logic; clk_50, resetn : in std_logic; LED : out std_logic_vector(7 downto 0)); end entity music; architecture fsm of music is -- Fill all the states ------------------Code here--------------------------- -- Declare state types here type state_type is (Silent, Ni, Sa, Re, Ga, Ma); ------------------Code here--------------------------- -- Declare all necessary signals here signal y_present : state_type := Silent; signal clock_music : std_logic := '1'; signal switch : std_logic_vector(7 downto 0); ------------------Code here--------------------------- -- Take the toneGenerator component component toneGenerator is port (toneOut : out std_logic; clk : in std_logic; LED : out std_logic_vector(7 downto 0); switch : in std_logic_vector(7 downto 0)); end component toneGenerator; ------------------Code here--------------------------- begin process(clk_50,resetn,clock_music) -- Fill sensitivity list variable count : integer range 0 to 32 := 0; variable y_next_var : state_type :=Silent; variable n_count : integer := 0; variable timecounter : integer range 0 to 1E8 := 0; begin y_next_var := y_present; n_count := count; case y_present is when Silent=> switch <= (0=>'0',others=>'0'); y_next_var := Sa; ------------------Code here--------------------------- ------------------Code here--------------------------- --assign the signal for switch which will be the input of toneGenerator component -----------------code here--------------------------- WHEN Sa => --if the machine in Sa state if((count = 1) or (count = 5) or (count = 9)) then y_next_var:=Sa; elsif((count = 2) or (count = 6) or (count = 10)) then y_next_var:=Ga; elsif((count = 16) or (count=31)) then y_next_var:=Ni; end if; switch <= (0 => '1', others => '0'); --assign the signal for switch which will be the input of toneGenerator WHEN Ga => if((count = 3) or (count = 7) or (count = 11)) then y_next_var:=Ga; elsif((count = 4) or (count = 8)) then y_next_var:=Sa; elsif((count = 14) or (count = 29)) then y_next_var:=Re; elsif(count = 12) then y_next_var:=Ma; end if; switch <= (2 => '1', others => '0'); ------------------Code here--------------------------- ------------------Code here--------------------------- WHEN Re => if((count = 23) or (count = 27) or (count = 19)) then y_next_var:=Re; elsif((count = 20) or (count = 24)) then y_next_var:=Ni; elsif((count = 15) or (count = 30)) then y_next_var:=Sa; elsif(count = 28) then y_next_var:=Ga; end if; switch <= (1 => '1', others => '0'); WHEN Ni => if((count = 21) or (count = 17) or (count = 25)) then y_next_var:=Ni; elsif((count = 18) or (count = 22) or (count = 26)) then y_next_var:=Re; elsif(count = 32) then y_next_var:=Silent; end if; switch <= (6 => '1', others => '0'); WHEN Ma=> y_next_var:=Ga; switch <= (3 => '1', others => '0'); END CASE; -- generate 4Hz clock (0.25s time period) from 50MHz clock (clock_music) if(clk_50='1' and clk_50' event) then if(timecounter>=6250000) then timecounter:=1; clock_music <= not clock_music; else timecounter:=timecounter+1; end if; end if; -- state transition if (clock_music = '1' and clock_music' event) then if (resetn = '1') then y_present <= Silent; count := 0; else y_present <= y_next_var; if(count>=32) then count :=0; else count := count + 1; end if; end if; end if; end process; TG: toneGenerator port map(toneOut,clk_50,LED,switch); -- instantiate the component of toneGenerator end fsm;

## RTL View:

## DUT Input/Output Format:

Switch 1 is used for reset input, PIN 89 is used to get 50MHz clock. For the toneGenerator, we use a variable to pass input to it. Output to the speaker is on PIN 1 and 8 LEDs corresponding to each note is also a part of output.

## RTL Simulation:

## Gate-level Simulation:

Attach the clearly visible screen-shot of Gate-level Simulation.

## Krypton board*:

## Observations*:

Correct output heard on speakers and correct LEDs glowed

## References:

You may include the references if any.

* To be submitted after the tutorial on ”Using Krypton.